1/3 INCH COLOR INDOOR IR DOME CAMERA, 24PCS IR LED

# S P E C I F I C A T I O N S

1/3 INCH COLOR INDOO IR DOME CAMERA, WITH 24PCS IR LED

MODEL NO.: C3833-56BBR1 (NTSC) / C3834-56BBR1 (PAL)

PICK-UP SIZE:                         1/3 INCH SONY CCD IMAGE SENSOR

SIGNAL MODE:                         NTSC                    PAL

VERTICAL FREQUENCY:                59.94 Hz                  50 Hz

PICTURE ELEMENTS:                   510 x 492 (NTSC)         500 x 582 (PAL)

HORIZONTAL FREQUENCY:             15.734 KHz               15.625 KHz

SYNC. MODE:                          INTERNAL

SCANNING SYSTEM:                   INTERLACE 2:1

MINIMUM ILLUMINATION:               0.02 LUX, F=1.2 (DAY), 0 LUX (IR ON)

HORIZONTAL RESOLUTION:            420 TV LINES

AUTO SHUTTER FUNCTION:            AUTO

S/N RATIO:                             MORE THAN 48dB

GAMMA CHARACTERISTIC:              r = 0.45

SYNCHRONOUS SYSTEM:              INTERNAL

GAIN CONTROL:                        AUTO

AUTO WHITE BALANCE:                 AUTO

BACK LIGHT COMPENSATION:           ON / OFF

AUTO ELECTRONIC SHUTTER:          NTSC: 1/60s to 1/100,000s, PAL: 1/50s TO 1/110,000s

VIDEO OUTPUT:                        COMPOSITE 1.0 Vp-p AT 75 OHMS

TRI-AXIS MOUNT:                       3-AXIS MECHANISM FOR FREE LENS ROTATION

IR LED:                                 24 IR LED

WAVE LENGTH:                         850 nm

BEAM SPREAD:                         35°

PROJECTION DISTANCE:                20 M

POWER REQUIREMENT:                 DC12V ± 10%

POWER CONSUMPTION:                 IR ON: 310mA (MAX.), IR OFF: 80mA (MAX.)

LENS FURNISHED:                       BUILT-IN f3.6mm BOARD LENS (OPTIONS: 6mm)

OPERATING TEMPERATURE:            -10°C  to  +45°C

DIMENSIONS :                           119(Φ) x 96.6(H)mm

WEIGHT :                               200g