1/3 INCH COLOR HI. RESO. IR CAMERA, 24PCS LED, 540 TVL

# S P E C I F I C A T I O N S

1/3 INCH COLOR HI. RESO. IR CAMERA, 24PCS LED, 540 TVL

MODEL NO.: C4853-62BABR1-2 (NTSC) / C4854-62BABR1-2 (PAL)

PICK-UP SIZE:                         1/3 INCH HI. RESO. HAD SENSOR

SIGNAL MODE:                         NTSC                    PAL

VERTICAL FREQUENCY:                59.94 Hz                  50 Hz

PICTURE ELEMENTS: (HxV)             768 x 494 (NTSC)          752 x 582 (PAL)

HORIZONTAL FREQUENCY:             15.734 KHz               15.625 KHz

SYNC. MODE:                          INTERNAL

SCANNING SYSTEM:                   2:1 INTERLACE

MINIMUM ILLUMINATION:               0.02 LUX, F=1.2 (DAY),   0 LUX (IR ON)

HORIZONTAL RESOLUTION:            540 TV LINES

S/N RATIO:                             MORE THAN 48dB (AGC OFF)

GAMMA CHARACTERISTIC:              r = 0.45

GAIN CONTROL:                        AUTO

WHITE BALANCE:                       AUTO

ELECTRONIC SHUTTER:                NTSC: 1/60s to 1/100,000s, PAL: 1/50s to 1/100,000s AUTO

LENS FURNISHED:                     BOARD LENS f3.6mm / F2.0 (OPTION: 4.3 / 6.0 / 8.0 / 12mm)

VIDEO OUTPUT:                        COMPOSITE 1.0 Vp-p AT 75 OHMS

IR LED:                                 24PCS IR LED

WAVE LENGTH:                         850 nm

BEAM SPREAD:                         35°

PROJECTION DISTANCE:                20 M

LED LIFE:                               6000 HRS

ILLUMINATOR ACTIVE LIMIT:             10 LUX

POWER REQUIREMENT:                DC12V ± 10%

POWER CONSUMPTION:                315mA MAX. (IR ON)

95mA MAX. (IR OFF)

OPERATING TEMPERATURE:           -10°C  to  +50°C

APPROVAL:                            IP 68

DIMENSIONS:                          62(Φ) x 142(L)mm

WEIGHT:                               450g